| KPI | Category | Description | Why should we use it ? |
| --- | --- | --- | --- |
| 1.)Total Number of Buses | Operational KPI | The Total Number of Buses refers to the number of individual buses required to execute the complete daily schedule across all routes, accounting for operational constraints like time windows, depot assignments, vehicle type (EV/diesel), and charging needs. | Suggests that the new model is more efficient and needs fewer buses to deliver the same service.Shows improved electrification and reduced emissions, reduces idle time and provides better fleet utilization |
| 2.)Charge Within Range | Operational KPI | Charge Within Range refers to the percentage or number of trips completed by electric buses without exceeding their available battery range or state of charge (SoC). It indicates whether buses can operate their assigned trips without requiring mid-trip charging or emergency interventions. | New model ensures better pre-trip planning and energy management. Efficient routing and SoC planning Reduced battery stress, longer battery life. |
| 3.)EPKM(Earning per KM) | Financial KPI | EPKM (Earning per Kilometer) measures the revenue earned by the bus service per kilometer operated. It includes earnings from ticket sales, passes, subsidies, and any other revenue sources linked to distance traveled. | New schedule generates more revenue per km. Shorter, more efficient routes may be more profitable. |
| 4.)Dead KM ratio |  | Dead Kilometers (Dead KM) refer to the distance a bus travels without carrying passengers | New schedule reduces wasted kilometers.Major cost savings and improved routing. |
| 5.)Max Charging Range Between Cycles | EV Specific KPI | Max Charging Range Between Cycles refers to the maximum distance an electric bus travels between two consecutive charging events in a given schedule. It indicates how far buses can operate before requiring a recharge. | It helps to compare Reduced number of charge cycles = less downtime. More trips completed on a single charge. |
| 6.)Crew Size 7.)No. Of shifts | Crew & Shifts KPI | Crew Size refers to the total number of staff (typically drivers) required to operate all scheduled trips in a day, including main shifts, reliefs, and standby personnel. Number of Shifts refers to the total number of driver shifts or duty blocks created to cover the entire service schedule across all buses. A shift includes the assigned trips, start and end time, breaks, and any deadhead moves. | A more efficient schedule minimizes idle crew time and reduces unnecessary shift overlaps.If a new schedule requires fewer crew members per km, it's easier to scale the system up sustainably |
| 8.)Large Breaks For Charging | EV Specific KPI | Large Breaks for Charging refers to the long idle durations built into the schedule specifically to allow EV buses to recharge their batteries during operations — either at depots or en-route charging stations. These breaks must be long enough to provide a meaningful state-of-charge (SoC) recovery and fit operational feasibility. | This KPI can help to recognise reduction in large, isolated charging breaks shows better integration with trip timing and crew shifts. |
| 9.)Crew Change Integration with Charging | EV Specific KPI | Crew Change Integration with Charging refers to how effectively the driver changeover points are aligned with planned EV charging events, minimizing idle time and improving operational efficiency. | A higher integration rate in the new model means better planning—less time wasted on separate stops. Avoids scheduling two separate breaks for charging and crew change. |
| 10.)Overlapping | Operational KPI | Overlapping in bus scheduling refers to the extent to which multiple trips or blocks cover the same route segments or time periods, causing redundancy or congestion in service. It measures inefficiencies in route planning, especially where buses run in close succession unnecessarily. | This KPI will help us to figure out how the model reduces unnecessary km — better for charge planning and SoC management and avoids sending multiple buses where one would suffice. |
| 11.)Max Headway Peak_M 12.)Max Headway Peak_E | Headways KPI | Max Headway Peak_M / Peak_E refers to the maximum time interval (in minutes) between two consecutive buses on the same route during the morning (Peak_M) and evening (Peak_E) peak hours, respectively. | We can use both of these KPI’s to ensure more frequent service when demand is high and reduced waiting time during evening rush |
| 13.)General KM | Operational Efficiency KPI | General KM refers to the total kilometers traveled by a bus while operating scheduled passenger trips (i.e., revenue service), excluding dead kilometers, depot trips, or positioning moves. | This KPI can help us to know how the new model reduces non-useful movement It helps to Lower non-productive mileage, extending vehicle life. |
| 14.)Night KM | Operational Efficiency KPI | Night KM refers to the total kilometers traveled by buses during the night period, typically defined as operations outside of peak daytime hours—often from around 10 PM to 5 AM (the exact time window may vary by transit agency). | This kpi can help in comparison how the New model limits nighttime operation — better energy and crew efficiency.Reducing night trips can lower accident risk and improve driver safety. |
| 15.)Steering Hours | Shift Related KPI | Steering Hours refer to the total number of hours during which a bus driver (crew) is actively operating or "steering" the vehicle, i.e., driving time spent in revenue or non-revenue movement. | This KPI can be used to show how new model uses crew more productively,improved compliance with labor rules,less idle or wasted time |
| 16.)Spread Hours | Shift Related KPI | Spread Hours refer to the total duration between a driver’s sign-on and sign-off time during a shift, including: - Driving (Steering) time - Breaks - Layovers - Idle time between trips | This KPI can be used to show increased scheduling efficiency and reduced idle or waiting time Shorter spreads mean fewer paid non-productive hours.. |
| 17.)Revenue Trips | Financial KPI | Revenue Trips are the bus trips that carry paying passengers and generate fare revenue. These include all scheduled trips operated along a service route between official stops or terminals, intended for passenger service. | This KPI can be used to significantly Identify more revenue-generating trips that increase farebox recovery. |
| 18.) Off-Peak Hours Headway | Service Availability and Scheduling KPIs | Off-Peak Hours Headway refers to the average time interval between consecutive buses on a route during non-peak hours (typically mid-morning, early afternoon, late evening, and weekends). | This KPI can be used to indicate improved service frequency even during low demand periods, which can enhance rider satisfaction and encourage more usage. |
| 19.)Average KM | Operational KPI | Average KM refers to the average number of kilometers covered per bus per day (or per duty/shift), across all deployed buses in a given schedule. |  |
| 20.)Total Vehicle KM | Operational KPI | Total Vehicle Kilometers (Total Vehicle KM) refers to the sum of all kilometers traveled by all buses over a given period (typically per day) in a schedule, including both revenue and non-revenue trips. | This KPI can be used to suggest inefficiencies, redundant trips, or increased charging/depot movement in EV schedules. |
| 21.)EV over limit | EV Specific KPI | EV Over Limit refers to the number of electric vehicle (EV) trips or duties that exceed the maximum battery range or energy capacity of the vehicle without a scheduled charging opportunity. It measures how often an electric bus is scheduled to operate beyond its feasible range, which can lead to operational failure unless corrected. | This KPI can help to know the fewer over-limit cases in the new model means better energy-aware scheduling. A higher EV Over Limit count = more risk of service failure or unscheduled depot returns. |